Hanwha Techwin is a leading supplier of advanced video surveillance solutions for IP-video, analog and hybrid systems. Building on the company's history of innovation, Hanwha Techwin is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-efficiency. Hanwha Techwin is committed to the continued development of innovative systems products for professional security applications.

For additional information, visit http://security.hanwhatechwin.com/

- PRODUCTS

  - EQUIPMENT

    - Manufacturer : Hanwha Techwin (http://security.hanwhatechwin.com/)

    - Model : TNO-L3030T

    - Alternates :	TNO-3030T

  - GENERAL DESCRIPTION

    - Thermal QVGA camera shall provide thermographic image so user can recognize object with heat even in dark scene

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265 and H.264 : Max. frame rates to 8fps

        - MJPEG : Max. frame rates to 8fps

      - The camera shall be able to configure up to 10 independent video stream per channel with differing encoding, quality, frame rate, resolution, and bit rate settings.

      - Resolution selections : 320x240

      - Simultaneous unicast access by up to 20 users

      - Multicast or unicast capable

      - Dynamic DNS (DDNS) support.

      - WiseStream Ⅱ, Dynamic GOV to efficiently manage bitrate of the video stream.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IK10, NEMA4X rated for protection against impacts

      - Provide video display on smart phone (iPhone, Android) to adjust viewing angle.

      - 32 privacy masking regions utilizing polygonal zones

    - Intelligence and Analytics – The camera shall have a suite of integral intelligent operations and analytic functions to include

      - Motion detection with eight definable detection areas, minimum / maximum object size definition.

      - Detection of logical events of specified conditions from the camera’s video input

    - Analytics events: Directional detection, Motion detection, Appear/Disappear, Enter/Exit, Loitering, Tampering, Virtual line, Audio detection, Temperature change detection, Sound classification, Shock detection

    - Interoperability – The camera shall be ONVIF Profile S / G / T compliant.

    - The camera shall possess the following further characteristics:

      - 8 different color palettes are supported : Whitehot, Blackhot, Rainbow, Rainbow2, Sepia, Red, Iron, Custom

      - Micro SD/SDHC/SDXC memory card and NAS recording options, with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - Alarm notification triggers:

          - Alarm input

          - Analytics events

          - Network disconnect

        - Available notification means upon trigger:

          - File upload via FTP and e-mail

          - Notification via e-mail

          - Record to local storage (Micro SD/SDHC/SDXC card or NAS)

          - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

          - Alarm output

      - POE capable

  - DETAILED SPECIFICATIONS

    - Video

      - Image

        - Sensor : Uncooled micro bolometer

          - Pixels per sensor : 320(H) x 240(V)

          - NETD:  <60mK

          - Pixel Size : 12㎛

        - The following features with control settings shall be available:

          - Camera Title

            - Off / On (Displayed up to 85 characters)

            - W/W : English / Numeric / Special characters

            - China : English / Chinese / Numeric / Special characters

            - Multi-line (Max. 5lines), Color (Grey / Green / Red / Blue / Black / White), Transparency, Auto scale by resolution

          - Digital Image Stabilization : Support (Built-in gyro sensor)

          - Motion Detection : Off / On (8ea, 8 point polygonal zones)

          - Privacy Masking : Off / On (32ea, polygonal zones)

            - Color : Gray / Green / Red / Blue / Black / White

          - Image flip : Off / On

          - Image mirror : Off / On

          - Hallway view : 0° / 90° / 270°

          - Single Images : The camera shall support jpg file image screenshot and export.

          - Analytics

            - Analytics events: Directional detection, Motion detection, Appear/Disappear,

Enter/Exit, Loitering, Tampering, Virtual line, Audio detection,

Temperature change detection, Sound classification, Shock detection

          - Alarm I/O : Input 1ea / Output 2ea

          - Alarm Triggers : Analytics, Network disconnect, Alarm input

          - Alarm Events

            - File upload via FTP and e-mail

            - Notification via e-mail

            - Micro SD/SDHC/SDXC or NAS recording at Event Triggers

            - Handover(PTZ Preset, Send message by HTTP/HTTPS/TCP)

            - Alarm output

          - Pixel Counter : Support

          - Video Out for installation

            - BNC: CVBS, 1.0 Vp-p / 75Ω

            - Micro USB Type B

        - Lens : 13.7 mm fixed

          - Max. Aperture Ratio : F1.0

          - FoV : H: 16.0˚,  V: 12.0°, D: 20.0°

          - Min. Object Distance : 8m (26.25ft)

          - Lens Type : Board-in Type

          - Focus Control : Fixed focus

      - Video Streams

        - The camera shall be able to produce 10 video profiles, which may have the following properties:

          - Encoding type : H.265, H.264, MJPEG

          - Resolution : 320x240

          - Frame rate

            - H.265 and H.264 : Max.8fps

            - MJPEG : Max. 8fps

          - Smart Codec : Manual(5ea area), WiseStream Ⅱ

          - Bit rate control

            - H.265 and H.264 : target bitrate level control, constant bit rate (CBR) or variable bit rate (VBR)

            - MJPEG : quality level control, variable bit rate (VBR)

          - Number of multi-streaming profiles : Up to 3 profiles

          - Streaming method : Unicast / Multicast

          - Simultaneous users (total) : 20 maximum (unicast)

      - Storage and Recording

        - The camera shall have onboard SD card storage.

          - Card type : Micro SD/SDHC/SDXC

          - Capacity : Up to  256GB (1 slot)

          - Image content on the card shall have the ability to be downloaded to a selected destination.

        - NAS

      - Interoperability - Video streams shall be capable of supporting ONVIF protocol, profiles S / G / T.

      - Memory : 1GB RAM, 256MB Flash

    - Network

      - Connectivity : RJ-45(10/100BASE-T)

      - Protocols supported :

        - Transmission Control Protocol (TCP), Internet Protocol (IP) v4 and v6, User Datagram Protocol (UDP)

        - Configuration: Dynamic Host Configuration Protocol (DHCP)

        - Web services: Hypertext Transfer Protocol (HTTP), Secure HTTP (HTTPS)

        - Security: Secure Sockets Layer(SSL)/Transport Layer Security(TLS)

        - Network services: Address Resolution Protocol (ARP), Bonjour, Domain Name System (DNS), Internet Control Message Protocol (ICMP), Link Layer Discovery Protocol(LLDP), Network Time Protocol (NTP), Protocol Independent Multicast-Sparse Mode (PIM-SM), Simple Network Management Protocol (SNMP v1/2c/3 – MIB-2), Universal Plug and Play (UPnP)

        - Media: Real-Time Transport Protocol (RTP), Real-Time Control Protocol, Real-Time Streaming Protocol (RTSP), Secure Real-Time Transport Protocol(SRTP)

        - Multicast: Internet Group Management Protocol (IGMP)

        - Notifications: File Transfer Protocol (FTP), Simple Mail Transfer Protocol (SMTP)

        - Remote Access: PPPoE

      - DDNS

        - The camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

      - Quality of Service (QoS)

        - Layer 3 DSCP

      - Security features

        - HTTPS(SSL) Login Authentication

        - Digest Login Authentication

        - IP Address Filtering

        - User access log

        - 802.1X Authentication(EAP-TLS, EAP-LEAP)

        - Device Certificate(Hanwha Techwin Root CA, pre-installed)

      - Application Programming Interface

        - ONVIF Profile S / G / T, SUNAPI(HTTP API), Wisenet open platform

      - Discovery

        - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

    - Audio

      - Audio In

        - Selectable (Microphone IN/Line IN),

        - Supply voltage: 2.5VDC(4mA), Input impedance: approx. 2K Ohm

      - Audio Out : Line out,  Max output level: 1 Vrms

    - Electrical

      - Input Voltage : PoE (IEEE 802.3af, Class3), 24VAC, 12VDC

      - Power consumption

        - PoE : Max 8.7W, typical 7.5W

        - 12VDC : Max 7.8W, typical 6.7W

        - 24VAC : Max 9.2W, typical 8.0W

    - Mechanical And Environmental

      - Material : Aluminum

      - Dimensions (W x H)  : Ø147.51 x 375.3mm (Ø 5.81" x 14.78")

      - Weight : 2512g (5.538 lb)

      - Temperature And Humidity

        - Operating : -40°C ~ +60°C(-40°F ~ +140°F) / Less than 90% RH (Non-condensing)

        - Storage : -50°C ~ +60°C(-58°F ~ +140°F) / Less than 90% RH (Non-condensing)

      - Environmental Rating

        - IP66

        - IK10

        - NEMA4X

      - RAL Code

        - RAL9003

    - EMC & Safety

      - FCC 47 CFR Part 15 Subpart B

        - ANSI C63.4-2017 Class A

      - IC Regulation ICES-003 Issue 7

        - ANSI C63.4-2017 Class A

      - CE EMC-Directive 2014/30/EU

        - EN 55032:2015/A11:2020 Class A

        - EN 50130-4:2011

      - UK EMC-Regulations 2016/1091

        - BS EN 55032:2015/A11:2020 Class A

        - BS EN 50130-4:2011

      - VCCI-CISPR 32: Class A

      - AS/NZS CISPR32:2015 Class A

      - UL listed

      - CE EN IEC 63000:2018 (hazardous substances)

END OF SECTION

8