Exacq Technologies, headquartered in Indianapolis, Indiana, is a leading developer of open architecture, Video Management System (VMS) solutions for security and surveillance applications. Our exacqVision VMS client-server solutions are scalable from a small single camera solution to large scale corporate or campus systems with thousands of cameras. Real-time and recorded video can be viewed, managed, and configured from any location on the network.

For additional information, contact:

Exacq Technologies, Inc.
	11955 Exit Five Parkway
	Fishers, IN 46037  USA

Phone: +1 317 845-5710

Web: https://www.exacq.com

E-mail: exacqinfo@tycoint.com

STORAGE SEVER SYSTEM MANAGER

# DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 00 00	Electronic Safety and Security

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 19	Digital Video Recorders

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

Important Note to Security Systems Specifiers

CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant and related to the product referenced in this specification.

MasterFormat 2014:

27 20 00 	Data Communications

28 05 00 	Common Work Results for Electronic Safety and Security

28 13 00 	Access Control

28 13 16 	Access Control Systems and Database Management

28 16 00 	Intrusion Detection

28 16 33 	Intrusion Detection Control, GUI, and Logic Systems

28 23 00 	Video Surveillance

28 23 13	Video Surveillance Control and Management Systems

28 23 16 	Video Surveillance Monitoring and Supervisory Interfaces

28 23 19 	Digital Video Recorders

28 23 23 	Video Surveillance Systems Infrastructure

28 23 29 	Video Surveillance Remote Devices and Sensors

MasterFormat 2016:

27 15 01.xx 	Video Surveillance Communications Conductors and Cables

27 20 00 	Data Communications

28 05 00	Common Work Results for Electronic Safety and Security

28 05 xx 	Power Sources for Electronic Safety and Security

28 05 xx 	Servers, Workstations and Storage for Electronic Safety and Security

28 05 xx 	Storage Appliances for Electronic Safety and Security

28 05 xx.xx 	Network Video Recorders

28 05 xx 	Cyber Requirements for Electronic Safety and Security

28 05 xx 	Communications Equipment for Electronic Safety and Security

28 05 xx 	Systems Integration and Interconnection Requirements

28 05 xx.xx 	Electrical

28 05 xx.xx	Information

28 10 00 	Access Control

28 10 xx 	Access Control Software

28 20 00 	Video Surveillance

28 2x 00 	Video Management System

28 30 00 	Security Detection, Alarm, and Monitoring

28 3x 00 	Intrusion Detection

28 3x xx.xx 	Intrusion Detection Interfaces to Security Monitoring and Control

IPS NETWORK VIDEO RECORDER

      - GENERAL

  - SUMMARY

## Section includes a device to acquire, record, store, and display video signals from directly connected IP network video cameras and encoders.

## Related Requirements

      - 28 23 19 – Digital Video Recorders

      - 28 23 23 – Video Surveillance Systems Infrastructure

      - 28 23 29 – Video Surveillance Remote Devices and Sensors

  - REFERENCES

    - Abbreviations

      - HDD – Hard Disk Drive

      - IP - Internet Protocol

      - LDAP – Lightweight Directory Access Protocol

      - Mbps – Megabits per second

      - NVR – Network Video Recorder

      - POS – Point of Sale

      - PSIM – Physical Security Information Management

      - VMS - Video Management System

    - Reference Standards

      - Institute of Electrical and Electronics Engineers (IEEE) 802.3 standards

      - FCC – Code of Federal Regulations, Title 47, Part 15

      - ISO / IEC 14496 – 10 – MPEG-4, Part 10 (H.264)

      - Tested to UL standard IEC 62368-1:2018 and EN 62368-1:2020

      - CE

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

  - QUALIFICATIONS

    - Manufacturer shall be ISO 9001 certified with a minimum of three years’ experience in manufacturing digital storage equipment and associated interfaces.

  - LICENSES

    - The NVR shall have 1 (Professional or Enterprise) camera license included, with a 5-year SSA.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited five-year warranty and updates for device firmware and client and web software during the warranty period.

      - An extended support option shall be available.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Exacq Technologies, Inc.

11955 Exit Five Parkway

Fishers, IN 46037  USA

Phone: +1 317 845-5710

Web: https://www.exacq.com

E-mail: exacqinfo@tycoint.com

    - Model:		Q-Series Desktop

Note: Exacq offers an IP (only) version of this product.

    - Alternates: None

  - DESCRIPTION

    - The Network Video Recorder (“NVR”) shall be an appliance to acquire, record, store, and display video signals from directly connected IP network video cameras and encoders.

    - The NVR appliance hardware shall have the following characteristics:

      - Chassis: 		Desktop

      - Camera inputs

        - IP:		Up to 64 streams (throughput dependent) (Q-Series Desktop)

      - Storage capacity:	<4 TB> <8 TB> <12 TB> <18 TB> <24 TB> on a single HDD

<36 TB> <48 TB> on two HDDs

Exacq part numbers differentiated by number of IPS inputs and on-board storage capacity	 (all models have up to 64 IP video inputs, depending on licenses acquired):

| Model Number | OS | IP Streams | Storage: Total |
| --- | --- | --- | --- |
| IP01-04T-1QL | Linux | 64 | 4TB |
| IP01-04T-1QW | Windows | 64 | 4TB |
| IP01-08T-1QL | Linux | 64 | 8TB |
| IP01-08T-1QW | Windows | 64 | 8TB |
| IP01-12T-1QL | Linux | 64 | 12TB |
| IP01-12T-1QW | Windows | 64 | 12TB |
| IP01-18T-1QL | Linux | 64 | 18TB |
| IP01-18T-1QW | Windows | 64 | 18TB |
| IP01-24T-1QL | Linux | 64 | 24TB |
| IP01-24T-1QW | Windows | 64 | 24TB |
| IP01-36T-1QL | Linux | 64 | 36TB |
| IP01-36T-1QW | Windows | 64 | 36TB |
| IP01-48T-1QL | Linux | 64 | 48TB |
| IP01-48T-1QW | Windows | 64 | 48TB |

      - Video compression:

        - IP:				MJPEG, H.264, H.265

      - Video outputs:			2 x DisplayPort, 1 x HDMI, 1 x VGA (Q-Series Desktop)

      - Microphone input:		1 RCA connector

      - Audio outputs:			1 stereo (RCA connectors)

      - Alarms:

        - Inputs:			provision for 8 external TTL

        - Outputs:			provision for 4 external TTL, 1 external relay

      - Server characteristics:

        - Operating system:		Windows 11

Ubuntu Linux 24.04

        - Operating system drive:	256 GB SSD

        - Monitor outputs:		1 VGA + 1 HDMI + 2 Display Port

        - Processor:			i3

        - Memory:			8 GB (16GB optional)

        - Network:			2 x 2.5 Gbps, 2 x 10Gbps (upgrade sold separately)

        - USB ports:			8

          - USB 2.0 		2 front + 2 rear

          - USB 3.1 		2 front

          - USB 3.2		2 rear

      - Enclosure

        - Material:			Painted steel

        - Dimensions (l x w x h):

          - Desktop:	4 in x 13 in x 12.5 in (101.6 mm x 330.2 mm x 317.5 mm)

        - Weight:

          - Pounds (lbs):	11

          - Kilograms (kg):	4.99

      - Electrical

        - Input voltage:		100/240 VAC

    - Video Management System (“VMS”)

      - The NVR shall come pre-loaded with VMS server software.

      - The VMS server software shall provide the following features as a minimum:

        - System

          - One server connection per client

          - Browser-based viewing of live and stored video

          - Auto detection of supported cameras

          - Support for fish-eye and panoramic lens cameras

          - Client bandwidth throttling

          - Soft triggers

          - Pre and post alarm recording

          - Continuous motion, time, or alarm-based recording, configurable per camera

        - Live video view

          - Multiple monitor view support

          - PTZ control and presets

          - Digital PTZ control and presets

          - Motion and alarm indication

          - Event linking on discrete inputs

        - Search, playback, export, archive

          - Instant replay

          - Event search

            - Thumbnail views

            - Timeline views

          - Multi-camera playback

          - Export options

            - USB storage device

            - .AVI, .MOV, .MP4 or.EXE file

        - The NVR shall have the ability to support pre-loaded VMS software providing additional advanced functionality, including the following:

          - System

            - Server connections – up to 512 via a thick client interface or 16 via web client

            - Ability to specify minimum and maximum retention times on a per camera basis

            - Time-lapse recording

            - Extended storage

            - Archiving

            - Audit trail

            - Custom user groups

            - Intelligent search

            - E-mail notifications on system health

            - Enterprise level camera, server, and user management

            - LDAP and active directory support

          - Live view

            - Event linking on video, serial, and health events

            - Video wall support

            - Event-driven and time-based video switching

            - Camera groups

            - Multi-streaming

            - Event notifications

            - Map support, including hierarchical maps

            - Two-way audio

          - Search, playback, export, archive

            - Multiple camera export

The NVR comes with the exacqVision Professional software pre-loaded. Additional functionality is available through upgrade to exacqVision Enterprise VMS software.

    - User Interfaces – The NVR shall support both thick client browser-based and a mobile web client interface.

      - Thick client

        - Client software shall be downloadable at no charge from the NVR Manufacturer’s web site and be fully compatible with all available features of the VMS server software.

        - The client software shall be available for Windows, Apple iOS, and Linux operating systems.

      - Mobile web client

        - A free mobile application shall be available permitting remote view of live and recorded video through the NVR.

        - The mobile application shall support PTZ control and the monitoring and activation of alarms and triggers for the mobile device.

        - The mobile application shall be available for devices running Apple iOS, Google Android, Microsoft Windows, and Amazon Kindle Fire software.

        - The mobile application shall allow simultaneous interaction with multiple NVR devices from the same Manufacturer.

        - The web service supporting the mobile application shall size the video stream to accommodate both low bandwidth and high bandwidth networks.

  - PERFORMANCE

    - Compatibility

      - Video: 				The NVR shall be compatible with the list of IP camera 						manufacturing partners located at: 								https://exacq.com/integration/ipcams/

      - Access control:			The NVR shall be compatible with the list of Access 						Control Integration partners listed at: 								https://exacq.com/integration/access_control/

      - POS and retail analytics:		The NVR shall be compatible with the list of POS and 						retail analytic partners listed at: 									https://exacq.com/integration/retail_analytics/

      - PSIM:				The NVR shall be compatible with the list of PSIM 						partners located at: https://exacq.com/integration/psim/

      - Intrusion:				The NVR shall be compatible with the list of intrusion 						partners located at:

https://exacq.com/integration/intrusion/

      - Intercom and audio analytics: 	The NVR shall be compatible with the list of Intercom and 					audio analytics partners located at: 								https://exacq.com/integration/intercom-audio-analytics/

Specifier should complete the above sections to include manufacturers of those existing or new devices or software that will interface with the NVR.
A list of integrations from Exacq is available at https://exacq.com/integration.

    - Recording

      - Video throughput:

        - Windows:				350 Mbps

        - Linux:				400 Mbps

    - Display

      - Local client display rate:

        - Windows:				300 FPS @ 1080p

        - Linux:				700 FPS @ 1080p

  - ENVIRONMENTAL

    - Operating temperature:			40 – 95 degrees Fahrenheit (4.5 – 35 degrees C)

    - Power/heat load (Max):			100W / BTU/H 204.728498

  - OPTIONAL EQUIPMENT

    - The NVR shall have optional expansion capability for alarm inputs and outputs.

END OF SECTION

- EXECUTION

  - INSTALLATION

    - Contractor shall comply with all Manufacturer installation guidelines.

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - STORAGE

    - Hardware shall be stored in an environment where temperature and humidity are in the range specified by the hardware manufacturer.

END OF SECTION